Wire Protocol Transceiver Design

Report

October 8, 2025

Ahmed Mohamed Elsayed Tabbash 20P1076

Kareem younis foad Yehya 22P0136

# 1-Wire Protocol Transceiver Design Report

Contents

1-Wire Protocol Transceiver Design Report	1

1. Introduction	3

1.1 Background	3

1.2 Characteristics of 1-Wire Protocol	3

1.3 Supported Transactions and Modes	3

2. Literature Review	4

2.1 Survey of Existing Implementations	4

2.2 Common Design Approaches	4

2.3 Challenges in 1-Wire Implementation	4

2.4 Summary of Reviewed Work	5

3. System Design	5

3.1 Design Specifications	5

3.2 Overall Architecture	6

3.3 Detailed Block Diagram	6

3.4 Timing Parameters and Constants	7

3.5 Transmitter FSM Description	8

3.6 Receiver FSM Description	9

4. System Simulation	10

4.1 Simulation Environment	10

4.2 Test Case Descriptions	10

4.3 Simulation Results Summary	10

5. VHDL/Verilog Code	11

6. Simulation Waveforms	11

7. Discussion and Conclusion	15

7.1 Design Challenges	15

7.2 Known Limitations	15

7.3 Recommendations and Future Work	15

## 1. Introduction

### 1.1 Background

The 1-Wire protocol is a communication standard developed by Dallas Semiconductor (now Maxim Integrated) that enables low-speed, low-power data transfer between a master device and one or multiple slave devices over a single data line plus ground. It is widely adopted for sensor interfacing, identification devices, and memory modules due to its simplicity and cost-effectiveness.

### 1.2 Characteristics of 1-Wire Protocol

- Single Data Line: Uses one wire for data transmission and reception along with a ground reference.

- Half-Duplex: Data transmission occurs in one direction at a time, requiring precise bus management.

- No Dedicated Clock Line: Timing is based on carefully defined pulse widths generated by the master, which slaves use to synchronize internally.

- Master-Slave Architecture: One master controls all communication; slaves only respond to master commands.

- Device Identification: Each slave contains a unique 64-bit ROM code used for addressing.

### 1.3 Supported Transactions and Modes

- Reset and Presence: Communication begins with the master sending a reset pulse; slaves respond with a presence pulse indicating readiness.

- Write Time Slots: The master writes bits by pulling the bus low for specified durations, defining write-0 and write-1 slots.

- Read Time Slots: The master samples the bus at precise times during read slots initiated by pulling the line low.

- Standard and Overdrive Speeds: Default speed is approximately 15.4 kbps; can be increased up to 125 kbps with overdrive mode.

## 2. Literature Review

### 2.1 Survey of Existing Implementations

Research on hardware implementations of the 1-Wire protocol mainly focuses on finite state machine (FSM)-based designs, emphasizing timing accuracy and resource optimization. Most existing modules divide the transceiver into separate transmitter and receiver components:

- Transmitter Modules: Generate timed pulses to reset the bus, write bits, and release the line appropriately.

- Receiver Modules: Detect reset pulses, generate presence signals, and sample bits at precise intervals.

### 2.2 Common Design Approaches

- Bit-Banging Controllers: Use software or FSMs to manually toggle pins and sample signals, typically in microcontrollers or FPGA designs.

- FSM-Based Verilog/VHDL Designs: Use state machines to precisely time pulse widths based on system clock cycles.

- Clock Domain Synchronization: Synchronize the asynchronous data line into the FPGA clock domain to avoid metastability and sampling errors.

- Tri-State Control: Implement open-drain (open-collector) style tri-state buffers to share the single bus line without contention.

### 2.3 Challenges in 1-Wire Implementation

- Accurate timing resolution for pulse widths at µs scale within FPGA clock domains.

- Correct handling of bus contention between master and slaves.

- Reliable presence detection and recovery delays.

- Managing multiple slaves in a bus configuration (addressing and collision avoidance).

### 2.4 Summary of Reviewed Work

Most successful designs focus on modularity with separate Tx/Rx, configurable parameters for timing based on clock frequency, and clear testbench-driven verification strategies. Our design builds on these principles, implementing synthesizable, reusable Verilog code tailored for 100 MHz FPGA clocks.

## 3. System Design

### 3.1 Design Specifications

- Clock Frequency: 100 MHz (10 ns period)

- Bus Interface: Single bidirectional data line with tri-state control

- Functionality:

  - Master transmitter generating reset, presence detect, write-0, write-1 slots

  - Slave receiver detecting reset, sending presence, sampling bits

- Data Width: Byte-level transmission (8 bits per frame)

- Operating Modes: Master (transmit only) and Slave (receive only)

### 3.2 Overall Architecture

| Module | Responsibility | Interface |
| --- | --- | --- |
| one_wire_tx | Master transmit FSM for bus reset and writing | clk, rst_n, start, tx_byte, busy, done, one_wire_data (inout) |
| one_wire_rx | Slave receive FSM for presence pulse and reading | clk, rst_n, enable, one_wire_data (inout), presence_detect, rx_valid, rx_byte |
| one_wire_top | Top-level module integrating Tx and Rx with bus | clk, rst_n, start, tx_byte, tx_done, rx_valid, rx_byte |

### 3.3 Detailed Block Diagram

### 3.4 Timing Parameters and Constants

All timing parameters are derived from the 1-Wire standard specifications, converted to clock cycles at 100 MHz:

| Parameter | Value (µs) | Cycles (at 100 MHz) | Description |
| --- | --- | --- | --- |
| Reset Low (T_RSTL) | 480 | 48,000 | Master holds line low for reset |
| Reset High (T_RSTH) | 480 | 48,000 | Master releases line after reset |
| Presence Detect Low (T_PDL) | 60 | 6,000 | Slave pulls line low to signal presence |
| Write-1 Low (T_W1L) | 6 | 600 | Duration for writing a logical '1' bit |
| Write-0 Low (T_W0L) | 60 | 6,000 | Duration for writing a logical '0' bit |
| Sample Time (T_RDS) | 15 | 1,500 | Time after falling edge to sample data bit |

### 3.5 Transmitter FSM Description

The transmitter FSM is designed to:

- Generate reset pulse by pulling the line low for T_RSTL

- Release the line and wait for presence pulse window (T_RSTH)

- Sample presence pulse from slaves

- Transmit bits LSB-first with timing defined for write-0 and write-1

- Signal completion via the done flag

States:

| State | Description |
| --- | --- |
| IDLE | Wait for start signal |
| RST_LOW | Pull line low for reset |
| RST_REL | Release line after reset low |
| PDH_WAIT | Wait before sampling presence |
| PDL_SAMPLE | Sample slave presence pulse |
| REC_WAIT | Wait recovery before data |
| BIT_INIT | Initialize bit transmission |
| BIT_LOW | Drive line low for bit |
| BIT_WAIT | Wait for time slot completion |
| BIT_REC | Increment bit index |
| FINISH | Complete transmission |

### 3.6 Receiver FSM Description

The receiver module operates as a slave that:

- Detects reset pulses by monitoring falling edges on the bus

- Sends presence pulse by driving line low for T_PDL

- After presence, listens for falling edges to detect write slots

- Samples the data line after T_RDS to read bits

- Assembles received bits into a byte and asserts rx_valid

## 4. System Simulation

### 4.1 Simulation Environment

- Simulator: ModelSim/QuestaSim (recommended)

- Clock frequency: 100 MHz

- Simulation time unit: nanoseconds

- Testbenches: Separate for transmitter (one_wire_tx_tb), receiver (one_wire_rx_tb), and top-level (one_wire_top_tb)

### 4.2 Test Case Descriptions

| Testbench | Purpose | Key Inputs | Expected Output |
| --- | --- | --- | --- |
| one_wire_tx_tb | Verify master transmitter FSM | Reset, start, tx_byte (0xA5) | done asserted, bus pulses correct |
| one_wire_rx_tb | Verify slave receiver functionality | Reset, enable, simulated bus | Presence detect, rx_valid with rx_byte matching transmitted data |
| one_wire_top_tb | End-to-end test for integrated design | Reset, start, tx_byte (0xA5) | tx_done asserted, rx_valid asserted with correct rx_byte |

### 4.3 Simulation Results Summary

- Transmitter module successfully generates reset pulse, presence sampling, and transmits data bits according to timing.

- Receiver module correctly generates presence pulse and samples bits.

- Top-level simulation shows data transmitted by master received accurately by slave.

- Timing of signals verified to comply with 1-Wire protocol requirements.

- No metastability or bus contention observed in simulation.

## 5. VHDL/Verilog Code

The full synthesizable Verilog code is included in the appendix with comments and documentation.
Key files:

- one_wire_tx.v — Master transmitter FSM

- one_wire_rx.v — Slave receiver FSM

- one_wire_top.v — Top-level integration

- Testbenches: one_wire_tx_tb.v, one_wire_rx_tb.v, one_wire_top_tb.v

## 6. Simulation Waveforms

Typical waveforms include:

- Clock and reset signals.

- start pulse triggering transmission.

- Tri-state one_wire_data line driven low during reset and data slots.

- Presence pulse from slave detected after reset.

- Bit-level write slots with different low pulse durations for logical '1' and '0'.

- Receiver rx_valid asserted after successful reception.

- rx_byte matching transmitted byte 0xA5.

one_wire_tx_tb.v WaveForm + Transcript Output

PASS: TRANSMISSION COMPLETE

one_wire_rx_tb.v WaveForm + Transcript Output

PASS: RX CAPTURED A5

one_wire_top_tb.v  WaveForm + Transcript Output

PASS: TOP-LEVEL TX  RX SUCCESSFYL CAPTURED : 0XA5

## 7. Discussion and Conclusion

### 7.1 Design Challenges

- Accurate timing generation required careful cycle count calculations for µs-level timing at 100 MHz.

- Synchronization of the asynchronous single-wire bus into the synchronous FPGA clock domain was critical.

- Tri-state bus management required to avoid contention between master and slave.

- Ensuring presence pulse generation and detection aligns strictly with the 1-Wire protocol timing.

### 7.2 Known Limitations

- Fixed clock frequency requirement; changing clock frequency requires parameter recalculation.

- Only basic data transmission implemented; no higher-level 1-Wire ROM commands or device addressing.

- Single byte transmission per reset/presence cycle.

- Real-world electrical considerations such as line capacitance and noise not modeled in simulation.

### 7.3 Recommendations and Future Work

- Extend protocol to support full 64-bit ROM code addressing for multiple slaves.

- Implement error detection and recovery mechanisms.

- Port design to parameterized clock frequency for portability.

- Develop hardware test setup with physical 1-Wire devices to validate design.

- Explore integration of a software driver for command-level communication.